Distributor: The Global One Group

ULTRASONIC PARTS WASHER

MODEL: ULTRA-900 (Trade Show Model)

#### ULTRA-900 STANDARD FEATURES

- Stainless Steel Construction

- Ultrasonic Transducer & Generator

- Pneumatic Load Immersion Platform (Stainless Steel)

- Oil Weir Tank

- Electric Tank Heater/Inconel

- PLC Control

- NEMA 4 Electrical Enclosure with Push Button Controls/ E-Stop & Rotary Disconnect Switch

- Automatic Fluid Level Shut Off

- Manually Operated Lid Closure with Lift Support Springs

- Tank Drain

- Tank Insulation

(Stainless Steel: AISI Grade 304)

#### Sequence of Operation

- Load Parts							(Manual/Operator)

- Lower Load Platform 					(Initiated by Operator)

- Close Lid 							(Manual/Operator)

- Start Cycle		   	                                               (Initiated by Operator)

- Automatic Wash Process

- Open Lid	      	                                                           (Manual/Operator)

- Raise Load Platform 						(Initiated by Operator)

- Unload Parts 							(Manual/Operator)

Technical Specifications

Working Envelope

Load Platform Length						15 ¾”

Load Platform Width							35 ¼”

Working Height   							15 ¾”

Loading Height From Floor						39.4”

Max Load Weight							550 Lbs.

Solution Tank

Fluid Capacity							100 Gal.

Heater 									8 kW

Drain Valve 			 					1”

Ultrasonic System

Power									3000 W

Frequency								25 kHz

Oil Weir Tank

Fluid Capacity							18 Gal

Drain Valve								1”

Power Requirements

Voltage								230V 3Ph

Control Voltage 							24V/AC

Machine Footprint 
Length									38.2”

Width  								68.1”
Height 								66.9”

Dry Weight  								490 lbs.

ULTRA-900 Technical Features

Construction: The machine is manufactured in AISI 304 stainless steel.

Immersion System: During cleaning, the pneumatically operated lift platform lowers the workload into the tank and agitates with an up-and-down motion to force the cleaning fluid around and through the parts.

Ultrasonic System: A piezoelectric ultrasonic transducer will be mounted in a working sump to avoid interference with draining and provide maximum energy to the workpiece.

Electric Tank Heater: A digital thermostat controller allows the operator to adjust the heat setting to achieve the desired operating temperature. The heating element will automatically cycle on/off to maintain the set temperature. (Range 60-140°F)

Oil Weir: The Integrated water system channels accumulated oil from the surface to a compartment next to the solution tank.

Fluid Level Sensor: The pump and heat source will automatically shut off to prevent damage until the solution reaches adequate levels.

Cabinet Lid: The manually operated lid is equipped with pneumatic lift springs for effortless operation.

Thermal Insulation: All vertical tank surfaces and lid are insulated and covered with stainless steel panels

Electrical Panel: NEMA 4 enclosure with PLC, illuminated push button controls, lights, thermostat control, overload protection, E-stop, and rotary main power switch. (IP 55/Painted RAL 7032)